Tender Specification Repeater Power Supply DC 52100

Place the cursor in your Web browser's address bar and scan the bar code. You find directly into the right service web portal. 
DRAGO simple – reliable

Repeater Power Supply

Powering and Isolation of 2- and 3-wire Transmitters,

slim housing for DIN-rail mounting, width 6.2 mm, protection class: IP 20

Input				0 - 20 mA, 4 - 20 mA, 0 - 10 V, 2 - 10 V
				Transmitter supply voltage approx. 17 V, max. 35 mA
				Overload 50 mA

Output				0 - 20 mA, 4 - 20 mA
				0 - 10 V, 2 - 10 V

Load 600 Ω at 20 mA, or 2 kΩ at 10 V

Range setting			calibrated signal setting

Cut-off frequency		5 kHz, 100 Hz (switchable)

Transmission error		< 0.1 % full scale

Power supply			16.8 to 31.2 V DC, approx. 1.2 W, In-Rail-Bus

Protective separation		EN 61140, 3-Port isolation up to 300 V AC/DC between all circuits

Test voltage			3 kV, 50 Hz, 1 min., input against output against power supply

Working voltage		600 V AC/DC for overvoltage category II and pollution degree 2

Type				Repeater Power Supply DC 52100 ___	(see order code)

Supplier			DRAGO Automation GmbH, Berlin, Germany
				Phone: +49-30-409982-0, Fax: +49-30-409982-10, 
				email: info@drago-automation.de, internet: www.drago-automation.de

or equivalent

Order code:

| Repeater Power Supply, calibrated range selection | DC 52100 S |
| --- | --- |
| Repeater Power Supply, calibrated range selection, In-Rail-Bus for power supply | DC 52100 B |