Tender Specification Potentiometer Transducer DR 4310

Place the cursor in your Web browser's address bar and scan the bar code. You find directly into the right service web portal. 
DRAGO simple – reliable

Potentiometer Transducer, Zero/Span compensation, calibrated signal setting

Isolation and conversion of potentiometer position signals,

slim housing for DIN-rail mounting, width 12.5 mm, protection class: IP 20

Input				Potentiometer, 500 Ω – 100 kΩ
				3-wire connection
Output				± 20 mA, ± 10 mA, 0 - 20 mA, 4 - 20 mA, 0 - 10 mA, 2 - 10 mA
				± 10 V, ± 5 V, 0 - 10 V, 2 - 10 V, 0 - 5 V, 1 - 5 V
Range setting			simple selectable via DIP switches

Subsequent readjustment	zero/span potentiometers on the front panel

Cut-off frequency		10 kHz, switchable to 30 Hz

Transmission error		< 0.1 % full scale

Power Supply			Universal power supply for 20 - 253 V AC/DC

Protective separation		EN 61140, 3-Port isolation up to 600 V AC/DC between all circuits

Test voltage			4 kV, 50 Hz, 1 min., input against output against power supply

Working voltage		1000 V AC/DC for overvoltage category II and pollution degree 2

Type				Potentiometer Transducer DR 4310 AG   	
Supplier			DRAGO Automation GmbH, Berlin, Germany

Phone: +49-(0)30-409982-0, Fax: +49-(0)30-409982-10,

email: info@drago-automation.de, internet: www.drago-automation.de

or equivalent