Tender Specification Shunt/mV Isolation Amplifier DS 7200

Place the cursor in your Web browser's address bar and scan the bar code. You find directly into the right service web portal. 
DRAGO simple – reliable

Shunt/mV Isolation Amplifier, Zero/Span compensation, calibrated signal setting

Isolation and conversion of bipolar and unipolar mV-signals,

slim housing for DIN-rail mounting, width 12.5 mm, protection class: IP 20

Input				± 60 mV, ± 100 mV, ± 150 mV, ± 250 mV, ± 300 mV, ± 500 mV
				0 - 60 mV, 0 - 100 mV, 0 - 150 mV, 0 - 250 mV, 0 - 300 mV, 0 - 500 mV
Output				± 20 mA, ± 10 mA, 0 - 20 mA, 4 - 20 mA, 0 - 10 mA, 2 - 10 mA
				± 10 V, ± 5 V, 0 - 10 V, 2 - 10 V, 0 - 5 V, 1 - 5 V
Range setting			simple selectable via DIP switches

Subsequent readjustment	zero/span potentiometers on the front panel

Cut-off frequency		10 kHz, switchable to 30 Hz

Transmission error		< 0.1 % full scale

Power Supply			Universal power supply for 20 - 253 V AC/DC

Protective separation		EN 61140, 3-Port isolation up to 600 V AC/DC between all circuits

Test voltage			4 kV, 50 Hz, 1 min., input against output against power supply

Working voltage		1000 V AC/DC for overvoltage category II and pollution degree 2

Type				Shunt/mV Isolation Amplifier DS 7200 AG – ___ – ___        (see order code)

Supplier			DRAGO Automation GmbH, Berlin, Germany

Phone: +49-(0)30-409982-0, Fax: +49-(0)30-409982-10,

email: info@drago-automation.de, internet: www.drago-automation.de

or equivalent

Order code for customer specific settings:

|  |  |  | Input |  | Output |
| --- | --- | --- | --- | --- | --- |
| DS 7200 AG – |  |  | XX | – | XX |
| Input | ± 60 mV |  | 50 |  |  |
|  | 0 - 60 mV |  | 51 |  |  |
|  | ± 100 mV |  | 52 |  |  |
|  | 0 - 100 mV |  | 53 |  |  |
|  | ± 150 mV |  | 54 |  |  |
|  | 0 - 150 mV |  | 55 |  |  |
|  | ± 250 mV |  | 56 |  |  |
|  | 0 - 250 mV |  | 57 |  |  |
|  | ± 300 mV |  | 58 |  |  |
|  | 0 - 300 mV |  | 59 |  |  |
|  | ± 500 mV |  | 60 |  |  |
|  | 0 - 500 mV |  | 61 |  |  |
| Output | ± 10 V |  |  |  | 00 |
|  | 0 - 10 V |  |  |  | 01 |
|  | 2 - 10 V |  |  |  | 02 |
|  | ± 5 V |  |  |  | 03 |
|  | 0 - 5 V |  |  |  | 04 |
|  | 1 - 5 V |  |  |  | 05 |
|  | ± 20 mA |  |  |  | 06 |
|  | 0 - 20 mA |  |  |  | 07 |
|  | 4 - 20 mA |  |  |  | 08 |
|  | ± 10 mA |  |  |  | 09 |
|  | 0 - 10 mA |  |  |  | 10 |
|  | 2 - 10 mA |  |  |  | 11 |

Bold: Default setting